| Ministry of Agriculture, Food and Rural Affairs | Ministère de l’Agriculture, de l’Alimentation et des Affaires rurales |
| --- | --- |

| Woodstock Resource Centre 401 Lakeview Drive Woodstock, ON N4T 1W2 Phone: 519-537-6621 Direct: 519-537-8305 |  |
| --- | --- |

Economic Development Division

Date:	23 August 2010

To:	Ivan Milin, EcoSpace Engineering

From:	Christine Brown

Nutrient Management Field Crop Lead

Subject:	Follow up to SDTC Application #2010A – 1748R and Guelph meeting

In recent months several samples of “larvae processed” manure, at various stages through the process have been analyzed for macro and micro nutrient content, as well as some other parameters.  The results of these analyses are listed in the tables below both in “as-is” form and on a dry matter basis.  The results help to get a better understanding of the process and where changes can be made to enhance the final product.  One component of analysis that remains to be completed is the calculation of mass balance or in other words, the tracking of volumes from initial layer poultry manure to finish product and by-products.  The analysis results show interesting trends, but additional sampling would be required until a consistent process produced consistent results.  Consistency of end-use product is essential for ensuring potential customer satisfaction and regulatory requirements (i.e. Nutrient Management Act and Regulations).

Organic matter levels are high, but can not be easily compared to cattle manure where there more cellulose and lignin in the ration and there is significant bedding resulting in a much higher carbon:nitrogen (C:N) ratio.  A C:N ratio indicates how quickly the soil microbial populations will break down the carbon while utilizing the nitrogen as a food source during that process.  A C:N ratio of soil is generally about 12:1 and in soil microorganisms is about 8:1.  In manures where the C:N ratio is less than 10:1, where good growing conditions exist (warm temperatures and adequate moisture) the mineralization of the nitrogen would be rapid and availability to a grass type crop, such as turf or corn would be higher than with most other types of solid manure.

Table 1 shows the analysis results in a form that would be representative of application to the field.

Table 1 - Larvae Processed Poultry Manure on an As-Is Basis

| Type AS - IS BASIS | DM % | N % | NH4N ppm | P % | K % | Cu ppm | Zn ppm | Al ppm | Bo ppm | Ca % | Fe ppm | Mg % | Mn ppm | Na % | Su ppm | C:N ratio | % OM | pH |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | copper | zinc | aluminum | boron | calcium | iron | magnesium | manganese | sodium | sulphur |  |  |  |
| Fresh layer manure - May 28 | 25.7 | 2.03 | 2433 | 0.41 | 0.43 | 18 | 113 | 56 | 3 | 1.8 | 197.4 | 0.12 | 87.9 | 0.08 | 868 | 6 | 20.3 |  |
| Fertilizer after sieves - sample -day 5 | 48.0 | 3.58 | 4680 | 1.07 | 1.15 | 45 | 304 | 141 | 11 | 4.0 | 447 | 0.30 | 248 | 0.21 | 2056 | 6 | 38.0 |  |
| Left in sive - sample - day 5 | 44.0 | 3.82 | 4263 | 0.79 | 0.81 | 32 | 195 | 92 | 8 | 2.4 | 334 | 0.20 | 167 | 0.15 | 1623 | 5 | 34.7 |  |
| Larvae - samples - day 5 | 25.3 | 2.41 | 126 | 0.25 | 0.28 | 10 | 48 | 12 | bdl | 0.24 | 114 | 0.08 | 68 | 0.15 | 1356 | 5 | 20.0 |  |
| Pupae - samples - day 5 | 32.0 | 3.04 | 100 | 0.33 | 0.30 | 24 | 68 | 6 | bdl | 0.25 | 87 | 0.13 | 111 | 0.14 | 1600 | 5 | 25.3 |  |
| Feathers - sample- day 5 | 66.4 | 6.44 | 3923 | 1.31 | 1.47 | 99 | 341 | 175 | 12 | 4.2 | 571 | 0.38 | 301 | 0.24 | 4693 | 5 | 52.4 |  |
| bdl = below detectable level |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2 shows most of the same information converted to available nutrients in lbs/ton. Table 3 shows the same information as table 1, but on a dry matter basis so that an “apples-to-apples” comparison can be made of each material.

Table 2 - Larvae Processed Poultry Manure Value (lbs/ton on an As-Is Basis)

| Type AS - IS BASIS – lbs/ton VALUE | DM % | Total N | NH4N | Available N | P205 | K20 | Zn | Ca | Mg | Mn | Su | C:N ratio | % OM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | * |  |  | zinc | calcium | magnesium | manganese | sulphur |  |  |
|  |  | lbs/ton |  |  |  |  |  |  |  |  |  |  |  |
| Fresh layer manure - May 28 | 25.7 | 40.6 | 4.9 | 22 | 15 | 9 | 0.2 | 36 | 2.4 | 0.2 | 1.7 | 6 | 20.3 |
| Fertilizer after sieves - sample -day 5 | 48.0 | 71.6 | 9.4 | 38 | 39 | 25 | 0.6 | 80 | 6 | 0.5 | 4.1 | 6 | 38.0 |
| Left in sive - sample - day 5 | 44.0 | 76.4 | 8.5 | 40 | 29 | 18 | 0.4 | 48 | 4 | 0.3 | 3.2 | 5 | 34.7 |
| Larvae - samples - day 5 | 25.3 | 48.2 | 0.25 | 24 | 9 | 6 | 0.1 | 4.8 | 1.6 | 0.1 | 2.7 | 5 | 20.0 |
| Pupae - samples - day 5 | 32.0 | 60.8 | 0.20 | 30 | 12 | 7 | 0.1 | 5.0 | 2.6 | 0.2 | 3.2 | 5 | 25.3 |
| Feathers - sample- day 5 | 66.4 | 128.8 | 7.8 | 66 | 48 | 32 | 0.7 | 84 | 7.6 | 0.6 | 9.4 | 5 | 52.4 |

General Comments:

- Value of layer poultry manure is consistently among the highest of all livestock types

- Layer poultry manure is high in nutrients, but also has a very strong odour and is difficult to apply at a low application rate

- Value of micro-nutrients such as calcium, zinc, magnesium, manganese, sulphur etc is difficult to determine.

- C:N ratio indicates how quickly the nitrogen conversion from organic form to crop available form could occur under typical crop growing conditions.  The higher the C:N ratio the longer the conversion.  A C:N ratio of 30:1 or higher could result in short-term crop nitrogen deficiency (i.e. fields that seldom have manure or legume crops)

- Other factors that add value to the “larvae-processed manure:

  - Reduction in odour results in:

    - Opportunity to apply in/near urban areas

  - Change in composition from “toothpaste” to “coffee grounds” consistency results in:

    - Easier calibration with more uniform application rate

  - Reduction in moisture level results in:

    - Reduced transportation cost

    - Reduced storage cost

    - Reduced land application cost

  - Opportunity for value-added components:

    - Pupae are high protein (~19% crude protein on an as-is basis or almost 60% crude protein on a dry matter basis)

    - Ammonia that may be lost during the “processing”

    - Organic matter represents 78% over 1550 lbs/ton on a dry matter basis.

      - Carbon feeds the microorganism populations (diversity)

      - OM increases the nutrient-holding capacity of the soil (especially sand and clay)

      - OM increases the water-holding capacity of the soil (especially sand and clay)

      - OM increases soil structure and aggregate stability and helps prevent soil erosion

Table 3 - Larvae Processed Poultry Manure on a Dry Matter Basis

| Type Dry Matter Basis | DM % | N % | NH4N ppm | P % | K % | Cu ppm | Zn ppm | Al ppm | Bo ppm | Ca % | Fe ppm | Mg % | Mn ppm | Na % | Su ppm | C:N ratio | % OM |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Fresh layer manure - May 28 | 25.7 | 7.90 | 9467 | 1.60 | 1.67 | 70 | 440 | 218 | 12 | 6.9 | 768 | 0.47 | 342 | 0.32 | 3377 | 6 | 78.99 |
| Fertilizer after sieves - sample -day 5 | 48.0 | 7.46 | 9750 | 2.23 | 2.40 | 94 | 633 | 294 | 23 | 8.3 | 930 | 0.63 | 518 | 0.43 | 4283 | 6 | 79.17 |
| Left in sive - sample - day 5 | 44.0 | 8.68 | 9689 | 1.80 | 1.84 | 73 | 443 | 209 | 18 | 5.4 | 760 | 0.45 | 379 | 0.35 | 3689 | 5 | 78.86 |
| Larvae - samples - day 5 | 25.3 | 9.53 | 498 | 0.99 | 1.11 | 40 | 190 | 47 | bdl | 1.0 | 451 | 0.32 | 267 | 0.60 | 5360 | 5 | 79.05 |
| Pupae - samples - day 5 | 32.0 | 9.50 | 313 | 1.03 | 0.94 | 75 | 213 | 19 | bdl | 0.8 | 273 | 0.41 | 347 | 0.43 | 5000 | 5 | 79.06 |
| Feathers - sample- day 5 | 66.4 | 9.70 | 5908 | 1.97 | 2.21 | 149 | 514 | 264 | 18 | 6.3 | 861 | 0.57 | 454 | 0.37 | 7068 | 5 | 78.92 |